Arab Health 2020 Stand 27-30 January

Stand dimensions

Dimensions (Room Structure)

Outer – 4000w x 5000l x 2850h

Inner(projected Area) 3600w x 3600l x 2650h

Wall and ceiling thickness – 200mm

Top Curve Radius – 200mm inner 400mm outer

Door way – 2000w x 2200h top corner radius to 200mm

Ceiling/roof to be load bearing (12mm timber sheet covered by 9mm plasterboard+skim)

Timber joists to be of suitable size and spacing to support structure 180mm joists

Structure surfaces to be finished as following:

Outer Main shell – White – semi gloss (RAL Code TBA)

Inner walls – Steel Grey 2 (Ral Code TBA) – Paint – Matt

Inner Ceiling – Reference Black (Ral Code TBA) Paint - Matt

Front end wall and ceiling end cap pinstripe blue (c93m74y2k0)

Front Wall Surface (doorway) - Outer – Blue (c93m74y2k0)

Rear Wall Surface seating area – Semi-Gloss Blue (c93m74y2k0)

Immersive Logo Top front end cap - Black Lettering Version (155mm high)

Immersive Logo Side Left Top Centre - Black Lettering Version (155mm high)

Immersive Logo Side Right Top Centre - Black Lettering Version (155mm high)

Inner Floor – Steel Grey 2 (RAL code TBA) – Vinyl – Matt

Outer Front Under recess and seating area – Med Blue Carpet Tiles

Power – Centre Rear from hall roof to above stand ceiling

NO FLOOR OBSTRUCTIONS, MUST BE PERFECTLY SMOOTH